# SindenPS GunCon2 Firmware

# Table of Contents

Part I: Multi‑OS Setup Guide

1. Prerequisites & System Requirements

2. Windows Installation (Toolchain, Git/Make, avrdude)

3. Linux (Debian/Ubuntu) Installation

4. macOS (Homebrew) Installation

5. Get LUFA & Verify Paths

6. Firmware Project Layout & Makefile Configuration

7. Build (make) & Flash (avrdude)

8. Wiring & Functional Verification on PS2

Part II: Detailed Firmware Reference

1. Architecture Overview

2. USB Descriptor Reference

3. Interrupt Endpoint Behaviour

4. SerialPortSecondary Frame Format

5. Coordinate Scaling Engine

6. Button Mapping Engine (Active‑Low)

7. Pedal Input (D10) Debounce & Override

8. Calibration/Param Packets (0x21/0x09)

9. Main Loop Scheduling

10. Memory Footprint

11. UART Timing & Reliability

12. Build & Flash Flow (Summary)

13. Known Limitations

14. Extensions & Variants

15. Appendix A — Button Table

16. Appendix B — Serial Frame Examples

17. Appendix C — USB Report Examples

# Part I: Deluxe Multi‑OS Setup Guide

## 1. Prerequisites & System Requirements

• ATmega32U4 board (Pro Micro / Leonardo).

• USB to PS2.

• Pi TX → Pro Micro RX1; common GND.

• Pedal (momentary) → D10 → GND.

## 2. Windows Installation

Install AVR toolchain, Git for Windows (make), and avrdude. Verify:

avr-gcc --version

avrdude --version

make --version

## 3. Linux (Debian/Ubuntu) Installation

Install via apt:

sudo apt update

sudo apt install gcc-avr avr-libc avrdude make git

## 4. macOS (Homebrew) Installation

Install via Homebrew:

brew install avr-gcc avrdude make git

## 5. Get LUFA & Verify Paths

git clone https://github.com/abcminiuser/lufa.git

Set LUFA_PATH in Makefile:

LUFA_PATH := /home/youruser/lufa   # Linux/macOS

LUFA_PATH := C:/Users/YourName/lufa  # Windows

## 6. Firmware Project Layout & Makefile Configuration

Ensure Makefile Caterina port is set if desired:

CATERINA_PORT := /dev/ttyACM0   # Linux/macOS

CATERINA_PORT := COM5           # Windows

## 7. Build & Flash

Build:

make clean

make

Flash (Caterina bootloader):

avrdude -p m32u4 -c avr109 -P COM5 -b 57600 -D -U flash:w:gcon2_32u4.hex   # Windows

avrdude -p m32u4 -c avr109 -P /dev/ttyACM0 -b 57600 -D -U flash:w:gcon2_32u4.hex   # Linux/macOS

## 8. Wiring & Verification

• Pi TX → Pro Micro RX1 (pin 0).

• Pedal → D10 (PB6) → GND.

• GND common.

• Micro‑USB → PS2.

# Part II: Hyper‑Detailed Firmware Reference

## 1. Architecture Overview

Emulates Namco GunCon2 with LUFA; consumes SerialPortSecondary 6‑byte frames; scales X/Y; maps buttons; debounces pedal (D10); emits 6‑byte USB interrupt report matching real hardware.

## 2. USB Descriptor Reference

VID 0x0B9A, PID 0x016A, bcdUSB 1.10, EP0=8B; Interface 0xFF/0x6A; EP1 IN interrupt 8B @ 8ms. Report: [buttons_lo,buttons_hi,x_lo,x_hi,y_lo,y_hi]; buttons active‑low; (0,0) reserved for off‑screen.

## 3. Interrupt Endpoint Behaviour

On IN ready: copy current report; overlay progressive flag (paramMode bit 8) if required; write 6 bytes; clear IN.

## 4. SerialPortSecondary Frame Format

Bytes: [222, X(0..256), Y(0..256), B1, DPad, 223]. B1 (ACTIVE‑HIGH): bit0 Trigger, bit1 PumpAction, bit2 FrontLeft, bit3 FrontRight, bit4 BackLeft, bit5 BackRight. DPad (ACTIVE‑HIGH): bit0 Up, bit1 Down, bit2 Left, bit3 Right.

## 5. Coordinate Scaling Engine

X_out = ((X_in * 638) / 256) + 1

Y_out = ((Y_in * 238) / 256) + 1

Clamp inputs >256; never emit 0 to preserve off‑screen sentinel.

## 6. Button Mapping Engine (Active‑Low)

if (pressed) buttons &= ~(1u << bit);  // press (active‑low)

else        buttons |=  (1u << bit);  // release

Bits: Trigger=13, Start=15, Select=14, A=3, B=2, C=1, D‑pad U/R/D/L=4/5/6/7.

## 7. Pedal Input (D10) Debounce & Override

PB6 input with pull‑up; momentary pedal to GND. 3‑count stability filter. While pressed, force A (bit 3) active‑low.

## 8. Calibration / Param Packets (0x21/0x09)

Class Interface OUT (bmRequestType=0x21, bRequest=0x09, len=6). Payload: [Xlo,Xhi,Ylo,Yhi,ModeLo,ModeHi]. Store paramX, paramY, paramMode. Bit 8 of paramMode = progressive. Optional X/Y offset application supported.

## 9. Main Loop Scheduling

USB task → pedal debounce → parse one serial frame → button map → pedal override → IN send. Ensures coherent per‑frame updates and low latency.

## 10. Memory Footprint

Globals: report(6B), debounce(3B), params(6B), serial buffer(5B). Flash ≈ 9–11 KB; RAM < 1 KB.

## 11. UART Timing & Reliability

UART1 115200 8N1; seek header 222; read 5 bytes; validate footer 223. No CRC (GunCon2‑like). Dropped frames only cause transient stutter.

## 12. Build & Flash Flow (Summary)

# Build

make clean

make

# Flash (Windows)

avrdude -p m32u4 -c avr109 -P COM5 -b 57600 -D -U flash:w:gcon2_32u4.hex

# Flash (Linux/macOS)

avrdude -p m32u4 -c avr109 -P /dev/ttyACM0 -b 57600 -D -U flash:w:gcon2_32u4.hex

## 13. Known Limitations

• No off‑screen generation unless explicitly forced.

• No multi‑gun pairing from a single MCU.

• SerialPortSecondary must be steady to prevent aim jitter.

## 14. Extensions & Variants

• Off‑screen reload: set (x,y)=(0,0) on a chosen condition.

• Multi‑gun: two Pro Micros with unique serials.

• Debug UART: optional TX0 prints for diagnostics.

• Apply calibration offsets (paramX/paramY) for specific titles.

## 15. Appendix A — Full Button Table

Active‑low GunCon2 bits:

Bit 15: Start

Bit 14: Select

Bit 13: Trigger

Bit 7 : D‑pad Left

Bit 6 : D‑pad Down

Bit 5 : D‑pad Right

Bit 4 : D‑pad Up

Bit 3 : A

Bit 2 : B

Bit 1 : C

## 16. Appendix B — Serial Frame Examples

X=128, Y=240, Trigger+Right pressed:

DE 80 F0 01 08 DF  # 222, X, Y, Buttons, DPad, 223

## 17. Appendix C — USB Report Examples

Example 6‑byte IN report:

00 F7 50 00 20 00  # buttons_lo buttons_hi x_lo x_hi y_lo y_hi